i60x Pallet Drawing

==========================================================

File “i60x pallet dwg, Aug23-2016, v1 -- FINAL PALLET DESIGN.pdf” is the final pallet design. The comments in this file are suggestions only.

==========================================================

.gko (keep out) shows mechanical outline of pcb

==========================================================

.gm1 has green rectangles that show how connectors overhanging edge of pcb

==========================================================

.gm15 shows box around the connector overhang and the pcb (3783mils x 2090mils)

==========================================================

.gm9 shows route- out and break-away tab (solid light blue areas) and v-score (thin light blue lines).

==========================================================

SUGGESTED drawing shows v-score and routing. Note that routing on i60x pallet is slightly different from routing on i240 pallet.

==========================================================

SUGGESTED Pallet Drawing, with grabber area at left and right

==========================================================

Pallet size is 8,226 mils wide x 10,690 mils high, as noted below.